Brine Tank Assemble Steps:

| 1. All Parts: Brine Tank, Tank Cover, Salt Grid, 3 Legs, Float Assemblage, Float Tube Assemblage, Brine Hose. | 2. Snap legs into base. | 3. Three legs with base. |
| --- | --- | --- |
| 4. Bend tabs on base away from legs to allow easier installation. | 5. Insert float tube into base with hole for tubing facing outward. | 6. Install base & float tube into tank aligning the hole in the float tube with the hole in the tank. |
|  | Too High | 6” to 8” from Bottom |
| 7. Put the salt grid and float tube into brine tank. | 8. Float can be set at the level desired. Just slide rubber using to desired height. | 9. Float can be set at the level desired. Most system is 6” to 8” from bottom. |

| 10. Remove the nut & sleeve from top of float assemblage. | 11. Put float assemblage into float tube. | 12. Insert hose through hole into brine tank. |
| --- | --- | --- |
| 13. Slide nut onto hose. | 14. Insert support sleeve into hose. | 15. Insert hose into elbow fitting. Thread nut onto elbow hand tighten only. Verify all is aligned as shown. |
| 16. Install cover on float tube. | 17. Add salt into brine tank. 2 bags Max. | 18. Install cover on brine tank. |

The balance of the function is controlled by control valve.